BLUECTL2 USER INFO - SYSTEM OPERATION OVERVIEW

Step 1 - Rishi to Laser Driver (Power Supply Control)

Wireless Connection

Rishi establishes a wireless communication link with the Power Supply Unit (PSU) that drives the laser system.

Power Sequencing Requirements

Proper voltage sequencing is required to prevent damage to the laser driver circuitry.

Power ON Sequence

- Enable 5V rail

- Enable 12V rail

The 5V supply must be stabilized before enabling 12V to ensure proper logic initialization of the laser driver.

Power OFF Sequence

- Disable 12V rail

- Disable 5V rail

The 12V supply must be removed first to prevent backfeeding or unstable shutdown conditions.

Pending Integration

- Awaiting full operational sequence definition (Step 1–18) from the Block-level control documentation.

Step 2 - Power Meter Integration (Laser Output Monitoring)

Hardware

- Optical Power Meter: PM400 or PM100

- Manufacturer: Thorlabs

Purpose

The power meter monitors real-time optical output from the laser driver for:

- Calibration

- Stability verification

- Safety validation

- Closed-loop tuning (if implemented)

Software & OS Requirements

- Compatible only with Windows OS

- Currently operated via a dedicated Desktop PC

- Communication interface: USB (standard Thorlabs driver stack)

Step 3 - Slicer Software & System Communication

Software Environment

- Slicer application developed in MATLAB

- Generates control parameters for laser operation

Communication Architecture

- Wireless communication with DAMx Software Touchscreen UI

- Responsible for:

  - Transmitting operational parameters

  - Triggering execution sequences

  - Monitoring system feedback

External Hardware Integration

- Interfaces with Thorlabs power monitoring hardware

- Coordinates control logic with laser driver subsystem

BlueCtl2 User Info

BlueCtl2 is designed to run on any Linux based computer but it must be one which has a BLE (Bluetooth Low Energy) capability. My preferred way of doing this is with a Raspberry Pi 4 SBC (Single Board Computer) although any Raspberry Pi with a BLE capability would be suitable.

Details of how to connect to it and the commands available are below ..

Server name : boris.shef.ac.uk

Port : 51003 (note this is a different port to the one used by BlueCtl).

All commands are preceded by ‘#xxx’ where xxx is a 3 digit number of the number of bytes in the command to follow (there are exceptions to this however).

When initially connected to the BlueCtl2 program will respond with “HELLO” (without the #xxx described above).

The list of commands the user can send to BlueCtl2 is as follows ..

‘PING’ to which BlueCtl2 responds ‘PONG’. This can be used as a connectivity test.

‘BYE’ on receipt of which BlueCtl2 closes the network socket and returns to waiting for an incoming connection from a client.

‘BUILD’ to which BlueCtl2 responds with the build number i.e ‘BlueCtl2 build 10’.

‘SETCURRddddllcccc’

Where ..

‘dddd’ is the controller device number

‘ll’ is the laser number (currently controller devices control two lasers’

‘cccc’ is the current in milliamps as an integer.

So ‘SETCURR0000012500’ would tell controller device 0 to set laser 1 to 2500 mA.

On receipt of this BlueCtl2 will respond with ‘OK’ if this is done or ‘ERR’ if there is a problem.

Note that if the Pi isn’t connected to the controller device then it will connect before sending this command. But if the controller device is already connected then the command will be carried out far more quickly.

‘CONDEVdddd’

Where ..

‘dddd’ is the controller device number

This command instructs the Pi to connect to controller with the device number dddd

On receipt of this BlueCtl2 will respond with ‘OK’ if this is done or ‘ERR’ if there is a problem.

‘DISDEVdddd’

This command instructs the Pi to disconnect from the controller with the device number dddd. On receipt of this BlueCtl2 will respond with ‘DONE’.

‘CONALL’

When sent this command the controller will attempt to contact all of the devices in its address lookup table. Once this process is completed (which may take several minutes) then BlueCtl2 will respond with ‘DONE’.

‘DISALL’

When sent this command the controller will disconnect with any devices which it is connected to at that moment. Once this process is completed (which may take several minutes) then BlueCtl2 will respond with ‘DONE’.

‘BUILD’

When sent this command the controller will respond with the build number of the BlueCtl2 software it is using.

‘HOWCON’

When it receives this command the controller will list the device numbers it has been programmed to communicate with. This takes the form ..

Device 0000 (D8:47:8F:07:0F:A0) - CON

Device 0001 (D8:47:8F:07:16:ED) - NOT

Device 0002 (D8:47:8F:07:16:DE) - NOT

Where you see the device number , device hardware address and CON to indicate a device is connected or NOT to show that it isn’t currently connected.

See the code in my example ‘SampleControl2.py’ for an example of their usage.

Ian Wraith (i.wraith@sheffield.ac.uk)

25th January 2024